Combined Heating/Cooling Dashboard: Subplot Curve Guide

Figure described: combined_heating_cooling_dashboard.png

General mapping note

Duty-mapped means the heating sweep variable (blanket duty, 0-1) is linearly remapped onto the cooling x-domain only for overlay plotting. It does not convert heating physics into airflow physics.

Subplot 1 (Top-Left): Bed Temperature vs Cooling Airflow

Curve: bed temperature (cooling)

Definition: Average of cooling-mode bottom and top steady bed temperatures at each cooling airflow sweep point.

X-axis: Cooling airflow (L/min).

Y-axis: Bed temperature (C).

Curve: bed temperature (heating) (duty-mapped)

Definition: Average of heating-mode bottom and top steady bed temperatures across blanket duty sweep.

X-axis: duty mapped into cooling airflow axis for visual comparison only.

Y-axis: Bed temperature (C).

Subplot 2 (Top-Right): Capacity and Outlet Air with Separated Regime Scales

Curve: Cooling capacity

Definition: max(-Q_to_bed, 0) for cooling design points.

Axis: left y-axis, Cooling capacity (W).

Curve: Heating capacity (duty-mapped)

Definition: max(Q_to_bed, 0) for heating design points, plotted against duty-mapped x.

Axis: right-offset y-axis, Heating capacity (W).

Curve: outlet air temperature (cooling)

Definition: Cooling-mode aeration outlet temperature after column/conditioning model at each cooling airflow point.

Axis: right y-axis, Outlet air temperature (cooling) (C).

Curve: outlet air temperature (heating) (duty-mapped)

Definition: Heating-mode outlet air temperature across blanket duty sweep; in wall-blanket mode this is typically the ambient reference used by heating evaluation.

Axis: second right-offset y-axis, Outlet air temperature (heating) (C).

Subplot 3 (Bottom-Left): Electrical Power vs Bed Temperature

Curve: Heating power

Definition: Total heating electrical input power across heating sweep points.

X-axis: heating bed temperature (C).

Y-axis: Electrical power (W).

Curve: Cooling power

Definition: spot-cooler electrical power + assist-blower electrical power, plotted against cooling bed temperature.

Note: Under vertical frozen-bottle mode, spot-cooler power may be zero and fan power may vary only slightly.

Subplot 4 (Bottom-Right): Moisture Loss vs Power and Flow with Separated Power Scales

Curve: Cooling electrical power

Definition: cooling combined electrical power versus cooling moisture-loss sweep.

Axis: left y-axis, Cooling electrical power (W).

Curve: Heating electrical power (duty-mapped)

Definition: heating electrical power versus duty-mapped moisture axis.

Axis: right-offset y-axis, Heating electrical power (W).

Curve: Fan flow rate (cooling)

Definition: cooling airflow sweep values versus cooling moisture-loss sweep.

Axis: right y-axis, Fan flow rate (L/min).

Curve: Fan flow rate (heating) (duty-mapped)

Definition: heating overlay flow proxy from duty mapping for x/y contextual comparison only.

Axis: right y-axis, Fan flow rate (L/min).

Why scaling was changed

Cooling and heating curves have different physical domains and dynamic ranges. Dedicated axes were added to prevent one regime from visually flattening the other.